From:	Miles Alderman and Benjamin Fields

To: 	Dr. Wade-Espinosa

Date: 	April 21, 2022

Re:	Lab 2 Report

## Overview

The objective of this lab exercise was to cycle 2 pairs of LEDS through different on-off configurations at independent frequencies, utilizing the cooperative multitasking strategy.

## Tasks

Task 1: Pattern_1:

Hardware interfacing code for LED Pair 1: this task cycles through different states of activation for the green and red LEDs in sequential order. The frequency with which they are changed is handled by Task 2: Timing_1.

States:

The following states are different on-off combinations of which the two LEDs which are cycled though.

- t1state0_init: 		Initialization: Set both LEDs to off, move to t1state1 on next cycle

- t1state1_G1R0:		G_LED_1 ON	and	R_LED_1 OFF

- t1state2_G0R0: 		G_LED_1 OFF	and	R_LED_1 OFF

- t1state3_G0R1: 		G_LED_1 OFF	and	R_LED_1 ON

- t1state4_G0R0: 		G_LED_1 OFF	and	R_LED_1 OFF

- t1state5_G1R1: 		G_LED_1 ON	and	R_LED_1 ON

- t1state6_G0R0: 		G_LED_1 OFF	and	R_LED_1 OFF

Task 2: Timing_1:

Timing control for LED Pair 1/Pattern_1 cycling: this task decrements the COUNT_1 variable each pass through Task 2 that the DONE_1 variable is 0. Changing the starting number of ticks in TICKS_1 (and thus COUNT_1) changes the frequency of blinking. Since one pass through the delay Task occurs between each decrement of timing_1, the period between LED state changes is 1ms*TICKS_1.

States:

- t2state0_init: This is the initialization state that resets TICKS_1 to COUNT_1, clears DONE_1, and sets t2state to 1 so that it goes into the increment cycle.

- t2state1_decrement_C1: This is the state that decrements the COUNT_1 variable during each pass through Task 2.

Task 3: Delay:

This task creates the 1ms delay between LED update cycles. It has a loop that decrements from $0584 to 1, which based on the clock speed takes about 1ms. This task is completed after every time tasks 1-4 are run, thus allowing timing_1 or timing_2 to set approximate frequencies of the LEDs.

States:

- t3state0_init: initialization for TASK_3

- t3state1_delayloop: jumps to DELAY_1ms subroutine which has loop that creates delay of 1ms

Task 4: Pattern_2:

Hardware interfacing code for LED Pair 2, this task cycles through different states of activation for the green and red LEDs in sequential order. The frequency with which they are changed is handled by Task 5: Timing_2.

States:

The following states are different on-off combinations of which the two LEDs which are cycled though.

- t2state0_init: 		Initialization: Set both LEDs to off, move to t2state1 on next cycle

- t2state1_G1R0:		G_LED_2 ON	and	R_LED_2 OFF

- T2state2_G0R0: 	G_LED_2 OFF	and	R_LED_2 OFF

- T2state3_G0R1: 	G_LED_2 OFF	and	R_LED_2 ON

- T2state4_G0R0: 	G_LED_2 OFF	and	R_LED_2 OFF

- T2state5_G1R1: 	G_LED_2 ON	and	R_LED_2 ON

- T2state6_G0R0: 	G_LED_2 OFF	and	R_LED_2 OFF

Task 5: Timing_2:

Timing control for LED Pair 2/Pattern_2 cycling: this task decrements the COUNT_2 variable each pass through Task 4 that the DONE_2 variable is 0. Changing the starting number of ticks in TICKS_2 (and thus COUNT_2) changes the frequency of blinking. Since one pass through the delay Task occurs between each decrement of timing_2, the period between LED state changes is 1ms*TICKS_2.

States:

- T2state0_init: This is the initialization state that resets TICKS_2 to COUNT_2, clears DONE_2, and sets t2state to 1 so that it goes into the increment cycle.

- T2state1_decrement_C2: This is the state that decrements the COUNT_2 variable during each pass through Task 2.

Inter-Task Communication Variables

| Communication Variable | Description | Variable Cleared | Variable Set |
| --- | --- | --- | --- |
| DONE_1 | Is decrement of one period of LED pair 1 is done | Task_2: State 1 | Task_2: State 1 |
| DONE_2 | Is decrement of one period of LED pair 2 is done | Task_5: State 1 | Task_5: State 1 |
| COUNT_1 | Variable decremented for time-keep of LED pair 1 | Task_2: State 1 | Task_2: State 1 |
| COUNT_2 | Variable decremented for time-keep of LED pair 2 | Task_5: State 1 | Task_5: State 1 |

Finite State Machine Diagrams:

The following figures contain the finite state machine diagrams for Task 1-5:

Figure 1. Task_1: Pattern_1 Finite State Machine Diagram

Figure 2. Task_2: Timing_1 Finite State Machine Diagram

Figure 3. Task_3: Delay_1ms Finite State Machine Diagram

Figure 4. Task_4: Pattern_1 Finite State Machine Diagram

Figure 5. Task_5: Timing_2 Finite State Machine Diagram

### Source Code: See addended .pdf